IRIS_DSP 设计文档

- IRIS_DSP模块结构图

Figure 1 IRIS_DSP模块结构图（虚线部分 ）

- DSP时钟，复位信号

  - 时钟信号

    - 输入时钟

- iris_dsp_wclk

DSP主工作时钟，用于内部状态机和内部逻辑的控制

- iris_dsp_hclk

AHB/APB Bus读、写时钟。当进行AHB/APB Bus读、写操作时，需通过门控信号关闭时钟iris_dsp_wclk

- apb_csi_clk_data

Mipi图像数据输入时钟

    - 输出时钟

- ext_flash_clk

外部Flash读、写时钟，由DSP内部分频产生

  - 复位信号

    - 输入复位

- iris_dsp_wrstn

用于DSP内部状态机和内部逻辑的复位，同iris_dsp_wclk同步

- iris_dsp_hrstn

用于AHB/APB Bus的读、写逻辑复位，同iris_dsp_hclk同步

    - 输出复位

无

    - 内部复位

Figure 2 DSP内部复位信号

- DSP_TOP输入输出信号

Table 1 hs_iris_dsp_top 输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | iris_dsp_wclk | 1 | I | CRM | dsp working clock |
| 2 | iris_dsp_wrstn | 1 | I | CRM | dsp working reset |
| 3 | iris_dsp_hclk | 1 | I | cm0_sys_top | AHB bus clock |
| 4 | iris_dsp_hrstn | 1 | I | cm0_sys_top | AHB bus reset |
| 5 | iris_dsp_en | 1 | I | cm0_sys_top | dsp enable |
| 6 | iris_dsp_int2cpu | 2 | O | cm0_sys_top | interrupts to cpu. Bit 0: DSP Status Interrupt |
| 7 | iris_dsp_int2pad | 1 | O | IO | interrupt to pad |
| 8 | iris_dsp_mhaddr | 32 | I | cm0_sys_top | ahb address |
| 9 | iris_dsp_mhsel | 1 | I | cm0_sys_top | ahb select line |
| 10 | iris_dsp_mhwrite | 1 | I | cm0_sys_top | ahb write/read |
| 11 | iris_dsp_mhwdata | 32 | I | cm0_sys_top | ahb write data |
| 12 | iris_dsp_mhrdata | 32 | O | cm0_sys_top | ahb read data |
| 13 | iris_dsp_mhtrans | 2 | I | cm0_sys_top | ahb trans type |
| 14 | iris_dsp_mhburst | 3 | I | cm0_sys_top | ahb burst type |
| 15 | iris_dsp_mhsize | 3 | I | cm0_sys_top | ahb size tupe |
| 16 | iris_dsp_mhresp | 2 | O | cm0_sys_top | ahb response |
| 17 | iris_dsp_mhready | 1 | I | cm0_sys_top | ahb ready feedback |
| 18 | iris_dsp_mhreadyout | 1 | O | cm0_sys_top | ahb slave ready state |
| 19 | iris_dsp_paddr | 10 | I | cm0_sys_top | apb address |
| 20 | iris_dsp_psel | 1 | I | cm0_sys_top | apb select line |
| 21 | iris_dsp_penable | 1 | I | cm0_sys_top | apb enable |
| 22 | iris_dsp_pwrite | 1 | I | cm0_sys_top | apb write/read |
| 23 | iris_dsp_prdata | 32 | I | cm0_sys_top | apb write data |
| 24 | iris_dsp_pwdata | 32 | O | cm0_sys_top | apb read data |
| 25 | iris_dsp_pready | 1 | O | cm0_sys_top | apb ready |
| 26 | iris_dsp_pslverr | 1 | O | cm0_sys_top | apb error |
| 27 | ext_flash_spi_clk | 1 | O | IO | Flash Clock |
| 28 | ext_flash_spi_csn | 1 | O | IO | Flash Chip Select. Low Active |
| 29 | ext_flash_spi_di | 4 | I | IO | Flash Data In |
| 30 | ext_flash_spi_do | 4 | O | IO | Flash Data Out |
| 31 | ext_flash_spi_do_oen | 4 | O | IO | Flash Data In / Out Control |
| 32 | api_csi_rstn | 1 | I | CSI |  |
| 33 | apb_csi_clk_data | 1 | I | CSI | csi idi clk |
| 34 | apb_csi_csi_data | 32 | I | CSI | csi idi data |
| 35 | apb_csi_bytes_en | 2 | I | CSI | csi idi byte enable |
| 36 | apb_csi_data_en | 1 | I | CSI | csi idi data enable |
| 37 | apb_csi_data_type | 6 | I | CSI | csi idi DT |
| 38 | apb_csi_dvalid | 4 | I | CSI | csi idi data valid |
| 39 | apb_csi_ecc | 8 | I | CSI | csi idi data ECC |
| 40 | apb_csi_header_en | 1 | I | CSI | csi idi header enable |
| 41 | apb_csi_hvalid | 4 | I | CSI | csi idi data line sync |
| 42 | apb_csi_virtual_channel | 2 | I | CSI | csi idi VC |
| 43 | apb_csi_vvalid | 4 | I | CSI | csi idi frame sync |
| 44 | apb_csi_word_count | 16 | I | CSI | csi idi word_count |
| 45 | para_image_clk | 1 | I |  | Parallel Image Clock |
| 46 | para_image_frv | 1 | I |  | Parallel Image Frame Valid |
| 47 | para_image_liv | 1 | I |  | Parallel Image Line Valid |
| 48 | para_image_data | 10 | I |  | Parallel Image Data |
| 49 | isp_video_clk_o | 1 | O |  | Image Video Output --- Clock |
| 50 | isp_video_f_o | 1 | O |  | Image Video Output --- Frame Valid |
| 51 | isp_video_l_o | 1 | O |  | Image Video Output --- Line Valid |
| 52 | isp_video_data_o | 8 | O |  | Image Video Output --- Image Data |

- hs_iris_dsp_top内部寄存器定义

CPU通过APB Bus读、写DSP内部寄存器，从而获取DSP的工作状态。DSP内部寄存器位宽均为32 Bit。

DSP内部寄存器中包含部分特殊寄存器。一般情况下，此类寄存器不允许APB Bus进行读、写操作，只有在APB Bus Write/Read Enable情况下，才能对其进行操作。本章中，此类寄存器将添加标识---[Special]。

  - Template Flag: [0x4000_9000] ~ [0x4000_90FC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | RO | 32’h0 | Template Flag |

  - Template Encrypt Key: [0x4000_9100] ~ [0x4000_911C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Template Encrypt Key |

  - Template Encrypt Enable: [0x4000_9120]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:1] | R/W | 31’h0 | Not Used |
| [0] | R/W | 1’b0 | Template Encrypt Enable. High Active |

  - Device User Code Size[Special]: [0x4000_9124]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:0] | R/W | 24’h0 | Device User Code Size. By Byte |

  - Template Index Max Threshold[Special]: [0x4000_9128]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:0] | R/W | 16’h0 | Template Index Max Threshold. |

  - ISP Image Width & Height: [0x4000_912C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | ISP Image Height |
| [15:0] | R/W | 16’h0 | ISP Image Width |

  - ISP Image Output Video Width & Height: [0x4000_9130]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | ISP Image Output Video Height |
| [15:0] | R/W | 16’h0 | ISP Image Output Video Width |

  - ISP Image Left Eye Clip Window (For Encode) Point: [0x4000_9134]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Left Eye Clip Window --- Left Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Left Eye Clip Window --- Top Point |

  - ISP Image Left Eye Clip Window (For Encode) Point: [0x4000_9138]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Left Eye Clip Window --- Bottom Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Left Eye Clip Window --- Right Point |

  - ISP Image Right Eye Clip Window (For Encode) Point: [0x4000_913C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Right Eye Clip Window --- Left Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Right Eye Clip Window --- Top Point |

  - ISP Image Right Eye Clip Window (For Encode) Point: [0x4000_9140]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Right Eye Clip Window --- Bottom Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Right Eye Clip Window --- Right Point |

  - ISP Image Left Eye Clip Window (For Shutter Control) : [0x4000_9144]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Left Eye Clip Window --- Left Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Left Eye Clip Window --- Top Point |

  - ISP Image Left Eye Clip Window (For Shutter Control) : [0x4000_9148]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Left Eye Clip Window --- Bottom Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Left Eye Clip Window --- Right Point |

  - ISP Image Right Eye Clip Window (For Shutter Control): [0x4000_914C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Right Eye Clip Window --- Left Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Right Eye Clip Window --- Top Point |

  - ISP Image Right Eye Clip Window (For Shutter Control): [0x4000_9150]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R/W | 4’h0 | Not Used |
| [27:16] | R/W | 12’h0 | ISP Image Right Eye Clip Window --- Bottom Point |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:0] | R/W | 16’h0 | ISP Image Right Eye Clip Window --- Right Point |

  - ISP Rx Parameters: [0x4000_9154]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | rx_encode_k |
| [15:0] | R/W | 16’h0 | rx_k |

  - ISP Rx Gray Parameters: [0x4000_9158]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | rx set gray |
| [7:0] | R/W | 8’h0 | rx set gray range |

  - ISP Rx Statistial Parameters --- High: [0x4000_915C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:19] | R/W | 13’h0 | Not Used |
| [18:0] | R/W | 19’h0 | ISP Rx Statistial High |

  - ISP Rx Statistial Parameters --- Low : [0x4000_9160]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:19] | R/W | 13’h0 | Not Used |
| [18:0] | R/W | 19’h0 | ISP Rx Statistial Low |

  - ISP Rx Statistial Parameters --- Mid: [0x4000_9164]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:19] | R/W | 13’h0 | Not Used |
| [18:0] | R/W | 19’h0 | ISP Rx Statistial Low |

  - ISP Rx Sample Factor Bit : [0x4000_9168]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:2] | R/W | 30’h0 | Not Used |
| [ 1:0] | R/W | 2’h0 | ISP Rx Sample Factor Bit |

  - ISP Rx Max Ratio: [0x4000_916C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R/W | 20’h0 | Not Used |
| [11:0] | R/W | 12’h0 | ISP Rx Max Ratio |

  - ISP Rx Sensor Select : [0x4000_9170]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:5] | R/W | 28’h0 | Not Used |
| [3:0] | R/W | 4’h0 | ISP Rx Sensor Select |

  - Template Encode Rx Parameters of Pupil Radius Range: [0x4000_9190]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | pupil radius range, max Set to 8’d92 |
| [7:0] | R/W | 8’h0 | pupil radius range, min Set to 8’d10 |

  - Template Encode Rx Parameters of Iris Radius Range: [0x4000_9194]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | iris radius range, max. Name as ‘iris_r_max’ Set to 8’d120 |
| [7:0] | R/W | 8’h0 | iris radius range, min. Name as ‘iris_r_min’ Set to 8’d80 |

  - Template Encode Rx Parameters of Median[Special_01]: [0x4000_9198]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R/W | 16’h0 | Not Used |
| [7:0] | R/W | 8’h0 | median threshold for eyelash check Set to 8’d24 |

  - Template Encode Rx Parameters of Median[Special_01]: [0x4000_919C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:10] | R/W | 22’h0 | Not Used |
| [9:8] | R/W | 2’h0 | down sample size 2’b00: 3x3 2’b01: 5x5 2’b10: 7x7 2’b11: 9x9 Set to 2’b00 |
| [7:0] | R/W | 8’h0 | down sample hi-light threshold Set to 8’d192 |

  - Template Encode Rx Parameters of Median[Special_01]: [0x4000_91A0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:10] | R/W | 22’h0 | Not Used |
| [9:8] | R/W | 2’h0 | full sample size 2’b00: 3x3 2’b01: 5x5 2’b10: 7x7 2’b11: 9x9 Set to 2’b10 |
| [7:0] | R/W | 8’h0 | full sample hi-light threshold Set to 8’d240 |

  - Template Encode Rx Parameters of Fake Check[Special_01]: [0x4000_91A4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:26] | R/W | 6’h0 | Not Used |
| [25:16] | R/W | 10’h0 | fack_score, UFIX_2_8, [0.0, 4.0). max Set to 10’d400 |
| [15:10] | R/W | 6’h0 | Not Used |
| [9:0] | R/W | 10’h0 | fack_score, UFIX_2_8, [0.0, 4.0). min Set to 10’d250 |

  - Template Encode Rx Parameters of Integrator[Special_01]: [0x4000_91A8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:3] | R/W | 29’h0 | Not Used |
| [2] | R/W | 1’b0 | window size for gaze 1’b0: 5x5 1’b1: 9x9 Set to 1’b0 |
| [1] | R/W | 1’b0 | window size for iris 1’b0: 5x5 1’b1: 9x9 Set to 1’b1 |
| [0] | R/W | 1’b0 | window size for eyelid 1’b0: 5x5 1’b1: 9x9 Set to 1’b0 |

  - Template Encode Rx Parameters of Pupil Feature[Special_01]: [0x4000_91AC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R/W | 20’h0 | Not Used |
| [11:8] | R/W | 4’h0 | gm coefficient Set to 4’d4 |
| [7:4] | R/W | 4’h0 | Not Used |
| [3:0] | R/W | 4’h0 | min eyelash number Set to 4’d4 |

  - Template Encode Rx Parameters of Pupil Feature[Special_01]: [0x4000_91B0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:30] | R/W | 2’h0 | Not Used |
| [29:16] | R/W | 14’h0 | down sample gm range, UFIX_9_5. max Set to 14’d4096 (128.0) |
| [15:14] | R/W | 2’h0 | Not Used |
| [13:0] | R/W | 14’h0 | down sample gm range, UFIX_9_5. min Set to 14’d512 (16.0) |

  - Template Encode Rx Parameters of Pupil Feature[Special_01]: [0x4000_91B4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R/W | 24’h0 | Not Used |
| [7:0] | R/W | 8’h0 | down sample margin Set to (iris_r_min / 3) |

  - Template Encode Rx Parameters of Pupil Feature: [0x4000_91B8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | down sample radius range. max Set to (pupil_r_max / 3) |
| [7:0] | R/W | 8’h0 | down sample radius range. min Set to (pupil_r_min / 3） |

  - Template Encode Rx Parameters of Pupil Feature[Special_01]: [0x4000_91BC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:30] | R/W | 2’h0 | Not Used |
| [29:16] | R/W | 14’h0 | full sample gm range, UFIX_9_5. max Set to 14’d3072 (96.0) |
| [15:14] | R/W | 2’h0 | Not Used |
| [13:0] | R/W | 14’h0 | down sample gm range, UFIX_9_5. min Set to 14’d384 (12.0) |

  - Template Encode Rx Parameters of Pupil Feature[Special_01]: [0x4000_91C0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R/W | 24’h0 | Not Used |
| [7:0] | R/W | 8’h0 | full sample margin Set to (iris_r_min) |

  - Template Encode Rx Parameters of Pupil Feature: [0x4000_91C4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | full sample radius range. max Set to (pupil_r_max + 1) |
| [7:0] | R/W | 8’h0 | full sample radius range. min Set to (pupil_r_min – 1) |

  - Template Encode Rx Parameters of Iris Feature[Special_01]: [0x4000_91C8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:20] | R/W | 12’h0 | Not Used |
| [19:16] | R/W | 4’h0 | for dm_ex Set to 4’d5 |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:8] | R/W | 4’h0 | for sm_max Set to 4’d9 |
| [7:4] | R/W | 4’h0 | Not Used |
| [3:0] | R/W | 4’h0 | for sm_min Set to 4’d3 |

  - Template Encode Rx Parameters of Iris Feature[Special_01]: [0x4000_91CC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:3] | R/W | 29’h0 | Not Used |
| [2:0] | R/W | 3’h0 | eyelash pix number threshold Set to 3’d3 |

  - Template Encode Rx Parameters of Iris Feature[Special_01]: [0x4000_91D0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:16] | R/W | 8’h0 | gray range, max Set to 8’d224 |
| [15:8] | R/W | 8’h0 | Not Used |
| [7:0] | R/W | 8’h0 | gray range, min Set to 8’d16 |

  - Template Encode Rx Parameters of Iris Feature[Special_01]: [0x4000_91D4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:25] | R/W | 7’h0 | Not Used |
| [24:16] | R/W | 9’h0 | dif range, max Set to 9’d96 |
| [15:9] | R/W | 7’h0 | Not Used |
| [8:0] | R/W | 9’h0 | dif range, min Set to 9’d10 |

  - Template Encode Rx Parameters of Eyelid Feature[Special_01]: [0x4000_91D8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | r range shift. max Set to 8’d8 |
| [7:0] | R/W | 8’h0 | r range shift. min Set to 8’d4 |

  - Template Encode Rx Parameters of Find Arc[Special_01]: [0x4000_91DC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:20] | R/W | 12’h0 | Not Used |
| [19:16] | R/W | 4’h0 | a range. increase Set to 4’d1 |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:8] | R/W | 4’h0 | a range. max Set to 4’d15 |
| [7:4] | R/W | 4’h0 | Not Used |
| [3:0] | R/W | 4’h0 | a range. min Set to 4’d0 |

  - Template Encode Rx Parameters of Find Arc[Special_01]: [0x4000_91E0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:16] | R/W | 8’h0 | b range. increase Set to 8’d4 |
| [15:8] | R/W | 8’h0 | b range shift. max Set to 8’d240 |
| [7:0] | R/W | 8’h0 | b range shift. min Set to 8’d16 |

  - Template Encode Rx Parameters of Find Arc[Special_01]: [0x4000_91E4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:16] | R/W | 8’h0 | c range. increase Set to 8’d4 |
| [15:8] | R/W | 8’h0 | c range shift. max Set to 8’d136 |
| [7:0] | R/W | 8’h0 | c range shift. min Set to 8’d16 |

  - Template Encode Rx Parameters of Find Arc[Special_01]: [0x4000_91E8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:20] | R/W | 12’h0 | Not Used |
| [19:8] | R/W | 12’h0 | min energy Set to 12’d51 |
| [7:0] | R/W | 8’h0 | x step Set to 8’d4 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_91EC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R/W | 18’h0 | Not Used |
| [13:8] | R/W | 6’h0 | pupil circle, for feature & total Set to 6’d6 |
| [7:6] | R/W | 2’h0 | Not Used |
| [5:0] | R/W | 6’h0 | pupil circle, for feature_l ¦ r & total Set to 6’d16 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_91F0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R/W | 18’h0 | Not Used |
| [13:8] | R/W | 6’h0 | pupil circle, for feature & count Set to 6’d4 |
| [7:6] | R/W | 2’h0 | Not Used |
| [5:0] | R/W | 6’h0 | pupil circle, for feature_l ¦ r & count_l ¦ r Set to 6’d3 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_91F4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R/W | 20’h0 | Not Used |
| [11:8] | R/W | 4’h0 | pupil center scan step when raw search Set to 4’d2 |
| [7:0] | R/W | 8’h0 | min dif of gray Set to 8’d18 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_91F8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:16] | R/W | 8’h0 | pupil radius scan range when acu search Set to 8’d4 |
| [15:8] | R/W | 8’h0 | pupil center scan range when raw search. dy Set to 8’d6 |
| [7:0] | R/W | 8’h0 | pupil center scan range when raw search. dx Set to 8’d6 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_91FC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R/W | 18’h0 | Not Used |
| [13:8] | R/W | 6’h0 | iris circle, for feature & total Set to 6’d6 |
| [7:6] | R/W | 2’h0 | Not Used |
| [5:0] | R/W | 6’h0 | iris circle, for feature_l ¦ r & total Set to 6’d16 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_9200]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R/W | 18’h0 | Not Used |
| [13:8] | R/W | 6’h0 | iris circle, for feature & count Set to 6’d4 |
| [7:6] | R/W | 2’h0 | Not Used |
| [5:0] | R/W | 6’h0 | iris circle, for feature_l ¦ r & count_l ¦ r Set to 6’d3 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_9204]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R/W | 20’h0 | Not Used |
| [11:8] | R/W | 4’h0 | iris center scan step when raw search Set to 4’d2 |
| [7:0] | R/W | 8’h0 | min dif of gray Set to 8’d6 |

  - Template Encode Rx Parameters of Find Circle[Special_01]: [0x4000_9208]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:16] | R/W | 8’h0 | iris radius scan range when acu search Set to 8’d3 |
| [15:8] | R/W | 8’h0 | iris center scan range when raw search. dy Set to 8’d9 |
| [7:0] | R/W | 8’h0 | iris center scan range when raw search. dx Set to 8’d9 |

  - Template Encode Rx Parameters of Rect Trans[Special_01]: [0x4000_920C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | hi-light threshold Set to 8’d240 |
| [23:20] | R/W | 4’h0 | Not Used |
| [19:16] | R/W | 4’h0 | mean value * (1.0 – this) = boundary of eyelash Set to 4’d11 |
| [15:12] | R/W | 4’h0 | Not Used |
| [11:8] | R/W | 4’h0 | mean value * (1.0 + this) = boundary of reflection Set to 4’d11 |
| [7:0] | R/W | 8’h0 | mean value + this = boundary of reflection Set to 8’d40 |

  - Template Encode Rx Parameters of Rect Trans[Special_01]: [0x4000_9210]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | Not Used |
| [23:16] | R/W | 8’h0 | y shift for check eyelash lower Set to 8’d4 |
| [15:8] | R/W | 8’h0 | y shift for check eyelash upper Set to 8’d48 |
| [7:0] | R/W | 8’h0 | y shift for check eyelid Set to 8’d4 |

  - Template Encode Rx Parameters of CLAHE[Special_01]: [0x4000_9214]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:4] | R/W | 28’h0 | Not Used |
| [3:0] | R/W | 4’h0 | clahe clip limitation, (0, 7] Set to 4’d2 |

  - Template Encode Rx Parameters of Check Pupil Center[Special_01]: [0x4000_9218]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | full sample Set to 8’d32 |
| [7:0] | R/W | 8’h0 | down sample Set to 8’d16 |

  - Template Encode Rx Parameters of Check Circle[Special_01]: [0x4000_921C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:11] | R/W | 21’h0 | Not Used |
| [10:8] | R/W | 3’h0 | iris, [1, 7] Set to 3’d1 |
| [7:3] | R/W | 5’h0 | Not Used |
| [2:0] | R/W | 3’h0 | pupil, [1, 7] Set to 3’d1 |

  - Template Encode Rx Parameters of Check Focus: [0x4000_9220]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | min, mi quality, UFIX_1_15, [0.0, 1.0] Set to 16’d7864 (0.24) |
| [15:0 | R/W | 16’h0 | min, lo quality, UFIX_1_15, [0.0, 1.0] Set to 16’d5243 (0.16) |

  - Template Encode Rx Parameters of Check Focus: [0x4000_9224]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:0 | R/W | 16’h0 | min, hi quality, UFIX_1_15, [0.0, 1.0] Set to 16’d9830 (0.30) |

  - Template Encode Rx Parameters of Check Focus: [0x4000_9228]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:0 | R/W | 16’h0 | max, UFIX_1_15, [0.0, 1.0] Set to 16’d16384 (0.50) |

  - Template Encode Rx Parameters of Correct Focus[Special_01]: [0x4000_922C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | UFIX_1_15, 0.441400 Set to 16’d14464 |
| [15:0 | R/W | 16’h0 | UFIX_1_15, 0.729600 Set to 16’d23908 |

  - Template Encode Rx Parameters of Check Valid Pix Number: [0x4000_9230]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31] | R/W | 1’b0 | Not Used |
| [30:16] | R/W | 15’h0 | mi quality Set to 15’d6554 (0.4) |
| [15] | R/W | 1’b0 | Not Used |
| [14:0] | R/W | 15’h0 | lo quality Set to 15’d4915 (0.3) |

  - Template Encode Rx Parameters of Check Valid Pix Number: [0x4000_9234]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:15] | R/W | 17’h0 | Not Used |
| [14:0] | R/W | 15’h0 | hi quality Set to 15’d9830 (0.6) |

  - Template Encode Rx Parameters of Check Segmentation[Special_01]: [0x4000_9238]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:9] | R/W | 23’h0 | Not Used |
| [8:0] | R/W | 9’h0 | max (pupil iris center distance) / (iris_r – pupil_r), UFIX_1_8, [0, 1] Set to 9’d128 (0.5) |

  - Template Encode Rx Parameters of Check Segmentation: [0x4000_923C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:25] | R/W | 7’h0 | Not Used |
| [24:16] | R/W | 9’h0 | dialoation range, UFIX_1_8, [0, 1]. lo quality, max Set to 9’d205 (0.800) |
| [15:9] | R/W | 7’h0 | Not Used |
| [8:0] | R/W | 9’h0 | dialoation range, UFIX_1_8, [0, 1]. lo quality, min Set to 9’d32 (0.125) |

  - Template Encode Rx Parameters of Check Segmentation: [0x4000_9240]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:25] | R/W | 7’h0 | Not Used |
| [24:16] | R/W | 9’h0 | dialoation range, UFIX_1_8, [0, 1]. md quality, max Set to 9’d180 (0.700) |
| [15:9] | R/W | 7’h0 | Not Used |
| [8:0] | R/W | 9’h0 | dialoation range, UFIX_1_8, [0, 1]. md quality, min Set to 9’d38 (0.150) |

  - Template Encode Rx Parameters of Check Segmentation: [0x4000_9244]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:25] | R/W | 7’h0 | Not Used |
| [24:16] | R/W | 9’h0 | dialoation range, UFIX_1_8, [0, 1]. hi quality, max Set to 9’d154 (0.600) |
| [15:9] | R/W | 7’h0 | Not Used |
| [8:0] | R/W | 9’h0 | dialoation range, UFIX_1_8, [0, 1]. hi quality, min Set to 9’d51 (0.200) |

  - Template Encode Rx Parameters of Check Segmentation: [0x4000_9248]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:26] | R/W | 6’h0 | Not Used |
| [25:16] | R/W | 10’h0 | min margin, UFIX_2_8, [0.004, 2.0], Right Set to 10’d2 (0.004) |
| [15:10] | R/W | 6’h0 | Not Used |
| [9:0] | R/W | 10’h0 | min margin, UFIX_2_8, [0.004, 2.0], Left Set to 10’d1 (0.004) |

  - Template Encode Rx Parameters of Check Segmentation: [0x4000_924C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:26] | R/W | 6’h0 | Not Used |
| [25:16] | R/W | 10’h0 | min margin, UFIX_2_8, [0.004, 2.0], Bottom Set to 10’d4 (0.004) |
| [15:10] | R/W | 6’h0 | Not Used |
| [9:0] | R/W | 10’h0 | min margin, UFIX_2_8, [0.004, 2.0], Top Set to 10’d3 (0.004) |

  - Template Encode Rx Parameters of Check Gaze: [0x4000_9250]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R/W | 24’h0 | Not Used |
| [7:0] | R/W | 8’h0 | hi-light threshold Set to 8’d220 |

  - Template Encode Rx Parameters of Check Gaze[Special_01]: [0x4000_9254]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R/W | 20’h0 | Not Used |
| [11:8] | R/W | 4’h0 | threshold of center gray rate, UFIX_2_2, [0.0, 4.0) Set to 4’d8 (2.00) |
| [7:4] | R/W | 4’h0 | Not Used |
| [3:0] | R/W | 4’h0 | threshold of foreground rate, UFIX_2_2, [0.0, 4.0) Set to 4’d5 (1.25) |

  - Template Encode Rx Parameters of Check Gaze: [0x4000_9258]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | win a range, relative to pupil center, FIX_7_0. Bottom Set to 8’d0 |
| [23:16] | R/W | 8’h0 | win a range, relative to pupil center, FIX_7_0. Top Set to 8’d0 |
| [15:8] | R/W | 8’h0 | win a range, relative to pupil center, FIX_7_0. Right Set to 8’d0 |
| [7:0] | R/W | 8’h0 | win a range, relative to pupil center, FIX_7_0. Left Set to 8’d0 |

  - Template Encode Rx Parameters of Check Gaze: [0x4000_925C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R/W | 8’h0 | win b range, relative to pupil center, FIX_7_0. Bottom Set to 8’d0 |
| [23:16] | R/W | 8’h0 | win b range, relative to pupil center, FIX_7_0. Top Set to 8’d0 |
| [15:8] | R/W | 8’h0 | win b range, relative to pupil center, FIX_7_0. Right Set to 8’d0 |
| [7:0] | R/W | 8’h0 | win b range, relative to pupil center, FIX_7_0. Left Set to 8’d0 |

  - Template Encode Rx Parameters of Check Gaze: [0x4000_9260]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:20] | R/W | 12’h0 | Not Used |
| [19:16] | R/W | 4’h0 | win e rate Set to 4’d4 |
| [15:8] | R/W | 8’h0 | win e extending vertical pix Set to 8’d0 |
| [7:0] | R/W | 8’h0 | win e extending horizontal pix Set to 8’d0 |

  - Template Encode Rx Parameters of Check Gaze: [0x4000_9264]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31] | R/W | 1’b0 | Not Used |
| [30:16] | R/W | 15’h0 | reflect pix threshold. max_nma Set to 15’d0 |
| [15] | R/W | 1’b0 | Not Used |
| [14:0] | R/W | 15’h0 | reflect pix threshold. min_nma Set to 15’d0 |

  - Template Encode Rx Parameters of Check Gaze: [0x4000_9268]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31] | R/W | 1’b0 | Not Used |
| [30:16] | R/W | 15’h0 | reflect pix threshold. max_nmb Set to 15’d0 |
| [15] | R/W | 1’b0 | Not Used |
| [14:0] | R/W | 15’h0 | reflect pix threshold. min_nmb Set to 15’d0 |

  - Template Encode Rx Parameters of Check Open: [0x4000_926C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | lo quality: min right or left eyelid uncovered angle / 360 * 256 Set to 8’d16 (22.5) |
| [7:0] | R/W | 8’h0 | lo quality: max upper or lower eyelid covered angle / 360 * 256 Set to 8’d128 (168.0) |

  - Template Encode Rx Parameters of Check Open: [0x4000_9270]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | hi quality: min right or left eyelid uncovered angle / 360 * 256 Set to 8’d21 (29.5) |
| [7:0] | R/W | 8’h0 | hi quality: max upper or lower eyelid covered angle / 360 * 256 Set to 8’d106 (150.0) |

  - Template Encode Rx Parameters of Check Iris Gray: [0x4000_9274]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:8] | R/W | 8’h0 | max Set to 8’d192 |
| [7:0] | R/W | 8’h0 | min Set to 8’d16 |

  - Template Match Rx Parameters[Special_02]: [0x4000_9280]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:7] | R/W | 25’h0 | Not Used |
| [6:0] | R/W | 7’h0 | mask in win check, [0, 81] Set to 7’d64 |

  - Template Match Rx Parameters[Special_02]: [0x4000_9284]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:15] | R/W | 17’h0 | Not Used |
| [14:0] | R/W | 15’h0 | match valid pix threshold Set to 15’d6554 |

  - Template Match Rx Parameters[Special_02]: [0x4000_9288]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:15] | R/W | 17’h0 | Not Used |
| [14:0] | R/W | 15’h0 | match score threshold Set to 15’’d64 |

  - Device ID Rx Parameters[Special_04]: [0x4000_92A8] ~ [0x4000_92B4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Device ID. USB Used |

  - Device ID Rx Parameters[Special_04]: [0x4000_92B8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | USB Used |

  - Device ID Rx Parameters[Special_04]: [0x4000_92BC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Device Status. USB Used |

  - DSP Receive Command (One Cycle): [0x4000_9300]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R/W | 24’h0 | Not Used |
| [7:0] | W | 8’h0 | Receive Command. One Cycle 8’h01: Device Info Configure 8’h02: Device User Code Configure 8’h07: Template Encode Start 8’h04: Template Encode Stop 8’h0D: Template Match Start 8’h0E: One Template Remove 8’h0B: All Template Erase 8’h08: One Template Write To Flash 8’h19: One Template Read From Flash 8’h1A: Display Image Configure 8’’h1F: Display Image Start 8’h1C: Display Image Stop |

  - DSP Current Status: [0x4000_9304]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | RO | 24’h0 | Not Used |
| [7] | RO | 1’b0 | DSP Initial Finish. Hold |
| [6:4] | RO | 3’h0 | Not Used |
| [3:0] | RO | 4’h0 | DSP Current Status. 4’h1: Idle Status 4’h2: Device Info Configure Status 4’h3: Device User Code Configure Status 4’h4: Template Encode Status 4’h5: Template Encode Stop Status 4’h6: Template Match Status 4’h7: Template Remove Status 4’h8: All Template Erase Status 4’h9: Template Write To Flash Status 4’hA: Template Read From Flash Status 4’hB: Display Image Configure Mode |

  - DSP Command Status: [0x4000_9308]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:9] | RO | 23’h0 | Not Used |
| [8] | RO | 1’b0 | Template Write To/Read From Flash Finish |
| [7] | RO | 1’b0 | Template Remove/Erase Finish |
| [6] | RO | 1’b0 | All Template Match Finish. Not Accept |
| [5] | RO | 1’b0 | Template Match Accept |
| [4] | RO | 1’b0 | Template Encode Finish & Not Valid / Stop Finish |
| [3] | RO | 1’b0 | Template Encode Valid |
| [2] | RO | 1’b0 | Display Image Configure Finish |
| [1] | RO | 1’b0 | Device User Code Configure Finish |
| [0] | RO | 1’b0 | Device Info Configure Finish |

DSP Command Status寄存器中，状态值将持续，直至DSP收到新指令

  - DSP Error Status: [0x4000_930C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:5] | RO | 27’h0 | Not Used |
| [4] | RO | 1’b0 | Template Match Time Out |
| [3] | RO | 1’b0 | Template Encode Time Out |
| [2] | RO | 1’b0 | Template Index Overflow |
| [1] | RO | 1’b0 | Flash Write & Read Check Error |
| [0] | RO | 1’b0 | Flash Write & Read Time Out |

DSP Error Status寄存器中，状态值将持续，直至DSP收到新指令

  - DSP Receive Control: [0x4000_9310]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:10] | R/W | 22’h0 | Not Used |
| [9:8] | R/W | 2’h0 | Display Image Select. 2’b01: Logo 2’b10: Image A 2’b11: Image B |
| [7:6] | R/W | 2’h0 | Not Used |
| [5:4] | R/W | 2’h0 | Write(Configure) Display Image Select 2’b01: Logo 2’b10: Image A 2’b11: Image B |
| [3:1] | R/W | 3’h0 | Not Used |
| [0] | R/W | 1’b0 | Template Clear Enable (When Remove/Erase) 1’b1: Clear Enable 1’b0: Clear Disable |

  - DSP Receive Template Index: [0x4000_9314]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:17] | R/W | 15’h0 | Not Used |
| [16] | R/W | 1’b0 | Template Transfer BRAM A/B Select. 1’b0: Template BRAM A |
| [15:0] | R/W | 16’h0 | DSP Receive Template Index |

  - DSP Template Encode Control Register[0x4000_9318]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R/W | 18’h0 | Not Used |
| [13:8] | R/W | 6’h0 | Template Encode Error Mask |
| [7:6] | R/W | 2’h0 | Not Used |
| [5:4] | R/W | 2’h0 | Template Encode Quality Threshold. |
| [3:2] | R/W | 2’h0 | Template Encode Mode. Bit 0: Load Image Enable, High Active 1’b0: Load Image; 1’b1: Not Load Image (AHB Download Image) Bit 1: Template Encode Enable. High Active 1’b0: Location Only, 1’b1: Location & Encode |
| [1:0] | R/W | 2’h0 | ISP Image Clip Select. 2’b01: Left Eye; 2’b10: Right Eye |

  - DSP Template Match Control Register: [0x4000_931C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:20] | R/W | 12’h0 | Not Used |
| [19:4] | R/W | 16’h0 | Download Mode. Template B Threshold |
| [3:2] | R/W | 2’h0 | Not Used |
| [1:0] | R/W | 2’h0 | Template Match Mode 2’b01: Normal Mode. Template B Read From Flash 2’b10: Download Mode. Template B Download By Server |

  - DSP Special Register APB Bus Write/Read Enable: [0x4000_9320]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Special Register(Part 00) Write Enable |

只有在该寄存器值为32‘h22C3时，APB Bus才能对DSP Special Register(Part 00)进行写操作

  - DSP Special Register APB Bus Write/Read Enable: [0x4000_9324]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Special Register(Part 01) Write Enable |

只有在该寄存器值为32‘h22C3时，APB Bus才能对DSP Special Register(Part 01)进行写操做

  - DSP Special Register APB Bus Write/Read Enable: [0x4000_9328]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Special Register(Part 02) Write Enable |

只有在该寄存器值为32‘h22C3时，APB Bus才能对DSP Special Register(Part 02)进行写操作

  - DSP Special Register APB Bus Write/Read Enable: [0x4000_932C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Special Register(Part 03) Read Enable |

只有在该寄存器值为32‘h22C3时，APB Bus才能对DSP Special Register(Part 03)进行读操作

  - DSP Special Register APB Bus Write/Read Enable: [0x4000_9330]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:0] | R/W | 32’h0 | Special Register(Part 04) Write Enable |

只有在该寄存器值为32‘h22C3时，APB Bus才能对DSP Special Register(Part 04)进行写操作

  - Template Encode Result Register: [0x4000_9348]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R | 16’h0 | Not Used |
| [15:8] | R | 8’h0 | down sample pupil estimate center, UFIX_8_0. cy |
| [7:0] | R | 8’h0 | down sample pupil estimate center, UFIX_8_0. cx |

  - Template Encode Result Register[Special_03]: [0x4000_934C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R | 16’h0 | Not Used |
| [15:8] | R | 8’h0 | full sample pupil estimate center, UFIX_8_0. cy |
| [7:0] | R | 8’h0 | full sample pupil estimate center, UFIX_8_0. cx |

  - Template Encode Result Register: [0x4000_9350]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R | 4’h0 | Not Used |
| [27:16] | R | 12’h0 | pupil circle. UFIX_8_4. cy |
| [15:12] | R | 4’h0 | Not Used |
| [11:0] | R | 12’h0 | pupil circle, UFIX_8_4. cx |

  - Template Encode Result Register: [0x4000_9354]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R | 20’h0 | Not Used |
| [11:0] | R | 12’h0 | pupil circle, UFIX_8_4. r |

  - Template Encode Result Register[Special_03]: [0x4000_9358]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:4] | R | 29’h0 | Not Used |
| [2:0] | R | 3’h0 | pupil circle, UFIX_8_4. (pupil_q_o) |

  - Template Encode Result Register: [0x4000_935C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:28] | R | 4’h0 | Not Used |
| [27:16] | R | 12’h0 | iris circle. UFIX_8_4. cy |
| [15:12] | R | 4’h0 | Not Used |
| [11:0] | R | 12’h0 | iris circle, UFIX_8_4. cx |

  - Template Encode Result Register: [0x4000_9360]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R | 20’h0 | Not Used |
| [11:0] | R | 12’h0 | iris circle, UFIX_8_4. r |

  - Template Encode Result Register[Special_03]: [0x4000_9364]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:4] | R | 29’h0 | Not Used |
| [2:0] | R | 3’h0 | iris circle, UFIX_8_4. (iris_q_o) |

  - Template Encode Result Register: [0x4000_9368]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R | 8’h0 | Not Used |
| [23:16] | R | 8’h0 | eyelid upper. (eyelid_u_c_o) |
| [15:8] | R | 8’h0 | eyelid upper. (eyelid_u_b_o) |
| [7:4] | R | 4’h0 | Not Used |
| [3:0] | R | 4’h0 | eyelid upper. (eyelid_u_a_o) |

  - Template Encode Result Register[Special_03]: [0x4000_936C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R | 20’h0 | Not Used |
| [11:0] | R | 12’h0 | eyelid upper. (eyelid_u_e_o) |

  - Template Encode Result: [0x4000_9370]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:24] | R | 8’h0 | Not Used |
| [23:16] | R | 8’h0 | eyelid lower. (eyelid_l_c_o) |
| [15:8] | R | 8’h0 | eyelid lower. (eyelid_l_b_o) |
| [7:4] | R | 4’h0 | Not Used |
| [3:0] | R | 4’h0 | eyelid lower. (eyelid_l_a_o) |

  - Template Encode Result Register[Special_03]: [0x4000_9374]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:12] | R | 20’h0 | Not Used |
| [11:0] | R | 12’h0 | eyelid lower. (eyelid_l_e_o) |

  - Template Encode Result Register: [0x4000_9378]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R | 16’h0 | Not Used |
| [15:8] | R | 8’h0 | upper eyelid covered angle |
| [7:0] | R | 8’h0 | lower eyelid covered angle |

  - Template Encode Result Register: [0x4000_937C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R | 16’h0 | Not Used |
| [15:8] | R | 8’h0 | uncover left angle |
| [7:0] | R | 8’h0 | uncover right angle |

  - Template Encode Result: [0x4000_9380]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R | 24’h0 | Not Used |
| [7:0] | R | 8’h0 | mean value of iris gray |

  - Template Encode Result: [0x4000_9384]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:17] | R | 15’h0 | Not Used |
| [16:8] | R | 9’h0 | distance / (iris_r – pupil_r), UFIX_1_8, [0, 1] |
| [7:0] | R | 8’h0 | distance between pupil center and iris center |

  - Template Encode Result: [0x4000_9388]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:9] | R | 23’h0 | Not Used |
| [8:0] | R | 9’h0 | dialoation, UFIX_1_8, [0, 1] |

  - Template Encode Result: [0x4000_938C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:26] | R | 6’h0 | Not Used |
| [25:16] | R | 10’h0 | margin, UFIX_2_8, [0, 2]. Right |
| [15:10] | R | 6’h0 | Not Used |
| [9:0] | R | 10’h0 | margin, UFIX_2_8, [0, 2]. Left |

  - Template Encode Result Register: [0x4000_9390]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:26] | R | 6’h0 | Not Used |
| [25:16] | R | 10’h0 | margin, UFIX_2_8, [0, 2]. Bottom |
| [15:10] | R | 6’h0 | Not Used |
| [9:0] | R | 10’h0 | margin, UFIX_2_8, [0, 2]. Top |

  - Template Encode Result Register: [0x4000_9394]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R | 16’h | Not Used |
| [15:0] | R | 16’h0 | corrected focus: UFIX_1_15, [0.0, 1.0] |

  - Template Encode Result Register: [0x4000_9398]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:15] | R | 17’h0 | Not Used |
| [14:0] | R | 15’h0 | valid pix number |

  - Template Encode Result Register: [0x4000_939C]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:6] | R | 26’h0 | Not Used |
| [5:0] | R | 6’h0 | error flag, can not generate template |

  - Template Encode Result Register: [0x4000_93A0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:2] | R | 30’h0 | Not Used |
| [1:0] | R | 2’h0 | template quality |

  - Template Encode Result Register: [0x4000_93A4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R | 18’h0 | Not Used |
| [13:0] | R | 14’h0 | reason of score != 1 |

  - Template Encode Result Register: [0x4000_93A8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R | 18’h0 | Not Used |
| [13:0] | R | 14’h0 | reason of score != 2 |

  - Template Encode Result Register: [0x4000_93AC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R | 18’h0 | Not Used |
| [13:0] | R | 14’h0 | reason of score != 3 |

  - Template Match Result Register: [0x4000_93C0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:8] | R | 24’h0 | Not Used |
| [7:0] | R | 8’h0 | match accept angle |

  - Template Match Result Register: [0x4000_93C4]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:14] | R | 18‘h0 | Not Used |
| [13:0] | R | 14’h0 | match accept unmasked pixel number |

  - Template Match Result Register: [0x4000_93C8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R | 16’h0 | Not Used |
| [15:0] | R | 16’h0 | match accept template index |

  - Template Match Result Register: [0x4000_93D0]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:9] | R | 23’h0 | Not Used |
| [8:0] | R | 9’h0 | match accept template score |

  - ISP Gain Control: [0x4000_93E8]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:0] | RO | 16’h0 | ISP Gain Control |

  - ISP Shutter Control: [0x4000_93EC]

| Bit | R/W | Default value | Description |
| --- | --- | --- | --- |
| [31:16] | R/W | 16’h0 | Not Used |
| [15:0] | RO | 16’h0 | Template Shutter Control |

- DSP各指令操作流程

本节中，DSP内部寄存器地址，仅列出低16位(Bit[0] ~ [15])，其地址高16位均为0x4000

  - DSP Initial Sequence

虹膜锁上电启动或硬复位时，DSP将自动进行初始化操作，其流程图所示：

Figure 3 DSP Initial Sequence

  - DSP Configure Sequence

    - Device Info Configure Sequence

Figure 4 Device Info Configure Sequence

    - Device User Code Configure Sequence (Test Only)

Device User Code共64K Byte，在指令发送前，CPU通过AHB Bus写入DSP内Image BRAM.

AHB Bus Address: 32’h3000_0000 ~ 32’h3008_FFFF

Figure 5 Device User Code Configure Sequence

    - Display Image Configure Sequence

Display Image Configure用于修改开机画面等功能显示图片.CPU在发生Display Image Configure指令前，需先设置Register[0x9310]，以指定当前需要修改的具体图片

Figure 6 Display Image Configure Sequence

  - Template Encode Sequence

CPU在发送模板编码指令前，必须先设置Register[0x9318]，以指定DSP的编码模式，若不设置该Register，DSP将以上次M0的设置进行编码操作

DSP图像编码完成后，将模板的参数信息(分数等)写入Register[0x9348] ~ [0x93AC]，CPU可在收到中断信息后，读取参数信息，以判断模板是否有效

Figure 6 Template Encode Sequence

  - Template Match Sequence

Template Match分为Normal Mode和Download Mode，其中，Normal Mode从Flash中读取Template用于识别，适用于虹膜锁操作；Download Mode则通过AHB Bus下载Template进行识别操作，适用于USB操作。

CPU发送识别指令前，需先设置Register[0x931C]，以指定识别模式。若使用Download Mode，需同时指定AHB Bus下载的Template总数(最大限制为64，0 ~ 63)，同时，下载Template B(比对模板)时，需从地址32‘h3009_2000开始。

Normal Mode下，CPU检测到DSP模板识别成功后，读取Register[0x93C0] ~ [0x93CC]，以确定识别成功模板的Index和分数等信息

Download Mode下，CPU通过AHB Bus读取每次模板比对后的结果。AHB Bus Address: 32’h3009_4000 ~ 32’h3009_4040

Note: Template Match Download Mode, 仅适用于FPGA，ASIC无需进行测试

Figure 7 Template Match Sequence

  - Template Remove/Erase Sequence

Template Remove分为两种模式，由CPU在发送Template Remove/Erase指令前，通过Register[0x9310]进行设置。若CPU写入32‘h01，DSP在执行指令时，将同时删除Template Flag和Template Data；若CPU写入32‘h00，DSP在执行指令时，仅删除Template Flag；若CPU不设置该寄存器，DSP将延用上次CPU的设置。

CPU发送Template Remove指令前，需通过Register[0x9314]指定需要删除的Template Index，该值不能超出DSP所支持的最大模板范围。

DSP Template Remove Command: 32’h0E

DSP Template Erase Command:   32’h0B

Figure 8 Template Remove / Erase Sequence

  - Template Write/Read Sequence

    - Template Write

Template Write指令用于将DSP编码得到的模板数据或CPU下发的模板数据写入Flash中。CPU在发送指令前，需通过Register[0x9314]，指定模板写入的地址(Template Index)，以及读取的Template BRAM。

CPU通过AHB Bus向DSP下发模板数据

Template BRAM A: 32’h3009_0000 ~ 32’h3009_1FFF

Template BRAM B: 32’h3009_2000 ~ 32’h3000_3FFF

Figure 9 Template Write Sequence

    - Template Read Sequence

Template Read指令，用于从Flash中读取指定的模板数据。在DSP完成Template Read指令后，CPU可以通过AHB Bus读取Template BRAM，从而得到模板数据。

CPU通过AHB Bus从DSP读取模板数据

Template BRAM A: 32’h3009_0000 ~ 32’h3009_1FFF

Template BRAM B: 32’h3009_2000 ~ 32’h3000_3FFF

Figure 10 Template Read Sequence

- DSP_CTRL_TOP模块

dsp_ctrl_top模块用于AHB/APB Bus读、写操作，执行CPU发送的各指令。

  - DSP_CTRL_TOP模块结构图

Figure 11 dsp_ctrl_top模块结构图

  - DSP_CTRL_TOP模块输入输出信号

Table 2 dsp_ctrl_top模块输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | wclk | 1 | I | CRM | dsp working clock |
| 2 | wrstn | 1 | I | CRM | dsp working reset |
| 3 | hclk | 1 | I | cm0_sys_top | AHB bus clock |
| 4 | hrstn | 1 | I | cm0_sys_top | AHB bus reset |
| 5 | dsp_status_intr | 1 | O | cm0_sys_top | DSP Status Interrupt. High Active |
| 6 | hsel | 1 | I | cm0_sys_top | AHB Bus Select |
| 7 | htrans | 2 | I | cm0_sys_top | AHB Bus Transfer Type |
| 8 | hsize | 3 | I | cm0_sys_top | AHB Bus Data Size Type |
| 9 | hwrite | 1 | I | cm0_sys_top | AHB Bus Write Enable. High Active |
| 10 | haddr | 32 | I | cm0_sys_top | AHB Bus Write/Read Address |
| 11 | hwdata | 32 | I | cm0_sys_top | AHB Bus Write Data |
| 12 | hrdata | 32 | O | cm0_sys_top | AHB Bus Read Data |
| 13 | hready | 1 | I | cm0_sys_top | AHB Bus Ready Feedback |
| 14 | hresp | 2 | O | cm0_sys_top | AHB Bus Response. Set to 2’h0 |
| 15 | hreadyout | 1 | O | cm0_sys_top | AHB Bus Ready |
| 16 | penable | 1 | I | cm0_sys_top | APB Bus Enable |
| 17 | psel | 1 | I | cm0_sys_top | APB Bus Select |
| 18 | pwrite | 1 | I | cm0_sys_top | APB Bus Write Enable. High Active |
| 19 | paddr | 10 | I | cm0_sys_top | APB Bus Write/Read Address |
| 20 | pwdata | 32 | I | cm0_sys_top | APB Bus Write Data |
| 21 | prdata | 32 | O | cm0_sys_top | APB Bus Read Data |
| 22 | pready | 1 | O | cm0_sys_top | APB Bus Ready |
| 23 | pslverr | 1 | O | cm0_sys_top | APB Bus Slave Error. Set to 1’b0 |
| 24 | isp_rx_image_width | 16 | O | dsp_isp_top | ISP Image Width Threshold |
| 25 | isp_rx_image_height | 16 | O | dsp_isp_top | ISP Image Height Threshold |
| 26 | isp_rx_video_o_width | 16 | O | dsp_isp_top | ISP Output Video Width |
| 27 | isp_rx_video_o_height | 16 | O | dsp_isp_top | ISP Output Video Height |
| 28 | isp_rx_clip_win_select | 2 | O | dsp_isp_top | ISP Image Clip Window Select. 2’b01: Left Eye |
| 29 | isp_rx_clip_win_l_t | 12 | O | dsp_isp_top | ISP Left Eye Image Clip Window |
| 30 | isp_rx_clip_win_l_l | 12 | O | dsp_isp_top | ISP Left Eye Image Clip Window |
| 31 | isp_rx_clip_win_l_r | 12 | O | dsp_isp_top | ISP Left Eye Image Clip Window |
| 32 | isp_rx_clip_win_l_b | 12 | O | dsp_isp_top | ISP Left Eye Image Clip Window |
| 33 | isp_rx_clip_win_r_t | 12 | O | dsp_isp_top | ISP Right Eye Image Clip Window |
| 34 | isp_rx_clip_win_r_l | 12 | O | dsp_isp_top | ISP Right Eye Image Clip Window |
| 35 | isp_rx_clip_win_r_r | 12 | O | dsp_isp_top | ISP Right Eye Image Clip Window |
| 36 | isp_rx_clip_win_r_b | 12 | O | dsp_isp_top | ISP Right Eye Image Clip Window |
| 37 | isp_rx_total_num | 17 | O | dsp_isp_top |  |
| 38 | isp_rx_th_min | 17 | O | dsp_isp_top |  |
| 39 | isp_rx_th_mid | 17 | O | dsp_isp_top |  |
| 40 | isp_rx_th_max | 17 | O | dsp_isp_top |  |
| 41 | isp_rx_set_mid | 8 | O | dsp_isp_top |  |
| 42 | isp_rx_set_rng | 8 | O | dsp_isp_top |  |
| 43 | isp_rx_rng_min | 8 | O | dsp_isp_top |  |
| 44 | isp_rx_gain | 12 | O | dsp_isp_top |  |
| 45 | isp_rx_shutter_max | 13 | O | dsp_isp_top |  |
| 46 | isp_rx_shutter_min | 13 | O | dsp_isp_top |  |
| 47 | isp_rx_hl_max | 16 | O | dsp_isp_top |  |
| 48 | isp_rx_hl_min | 8 | O | dsp_isp_top |  |
| 49 | isp_rx_max_ratio | 12 | O | dsp_isp_top |  |
| 50 | isp_gain_ctrl | 16 | I | dsp_isp_top | ISP Image Gain Control |
| 51 | isp_gain_ctrl_valid | 1 | I | dsp_isp_top | ISP Image Gain Control Valid |
| 52 | isp_shut_ctrl | 16 | I | dsp_isp_top | ISP Image Shutter Control |
| 53 | isp_shut_ctrl | 1 | I | dsp_isp_top | ISP Image Shutter Control Valid |
| 54 | isp_image_sof | 1 | I | dsp_isp_top | ISP Image Frame Start |
| 55 | isp_image_eof | 1 | I | dsp_isp_top | ISP Image Frame End |
| 56 | isp_image_data | 10 | I | dsp_isp_top | ISP Image Frame Data |
| 57 | isp_image_dval | 1 | I | dsp_isp_top | ISP Image Frame Data Valid(768 * 768) |
| 58 | isp_image_accept | 1 | I | dsp_isp_top | ISP Image Frame Accept |
| 59 | flash_init_done | 1 | I | ext_flash_top | Flash Control Initial Finish |
| 60 | flash_finish | 1 | I | ext_flash_top | Flash Write/Read Finish |
| 61 | flash_rstn | 1 | I | ext_flash_top | Flash Software Reset. Low Active |
| 62 | flash_wr | 1 | O | ext_flash_top | Flash Write Start. One Cycle |
| 63 | flash_rd | 1 | O | ext_flash_top | Flash Read Start. One Cycle |
| 64 | flash_addr | 24 | O | ext_flash_top | Flash Write/Read Byte Address |
| 65 | flash_cnt | 16 | O | ext_flash_top | Flash Write/Read Page Count |
| 66 | flash_wr_req | 1 | I | ext_flash_top | Flash Write One Page Request |
| 67 | flash_wr_data | 4 | O | ext_flash_top | Flash Write Data |
| 68 | flash_wr_dval | 1 | O | ext_flash_top | Flash Write Data Valid |
| 69 | flash_rd_data | 4 | I | ext_flash_top | Flash Read Data |
| 70 | flash_rd_dval | 1 | I | ext_flash_top | Flash Read Data Valid |
| 71 | core_encrypt_init_start | 1 | O | dsp_core_top | Template Encrypt Initial Start |
| 72 | core_encrypt_init_finish | 1 | I | dsp_core_top | Template Encrypt Initial Finish |
| 73 | core_encrypt_en | 1 | O | dsp_core_top | Template Encrypt Enable |
| 74 | core_encrypt_key | 256 | O | dsp_core_top | Template Encrypt Key |
| 75 | core_encrypt_key_val | 1 | O | dsp_core_top | Template Encrypt Key Valid |
| 76 | core_encode_rstn | 1 | O | dsp_core_top | Template Encode Software Reset. Low Active |
| 77 | core_encode_start | 1 | O | dsp_core_top | Template Encode Start. One Cycle |
| 78 | core_encode_finish | 1 | I | dsp_core_top | Template Encode Finish |
| 79 | core_encode_valid | 1 | I | dsp_core_top | Template Encode Valid |
| 80 | core_rx_encode_reg00 | 16 | O | dsp_core_top | Template Encode Control |
| 81 | core_rx_encode_reg01 | 16 | O | dsp_core_top | Template Encode Control |
| 82 | core_rx_encode_reg02 | 16 | O | dsp_core_top | Template Encode Control |
| 83 | core_rx_encode_reg03 | 16 | O | dsp_core_top | Template Encode Control |
| 84 | core_rx_encode_reg04 | 16 | O | dsp_core_top | Template Encode Control |
| 85 | core_rx_encode_reg05 | 16 | O | dsp_core_top | Template Encode Control |
| 86 | core_rx_encode_reg06 | 16 | O | dsp_core_top | Template Encode Control |
| 87 | core_rx_encode_reg07 | 16 | O | dsp_core_top | Template Encode Control |
| 88 | core_rx_encode_reg08 | 16 | O | dsp_core_top | Template Encode Control |
| 88 | core_rx_encode_reg09 | 16 | O | dsp_core_top | Template Encode Control |
| 90 | core_rx_encode_reg0A | 16 | O | dsp_core_top | Template Encode Control |
| 91 | core_rx_encode_reg0B | 16 | O | dsp_core_top | Template Encode Control |
| 92 | core_rx_encode_reg0C | 16 | O | dsp_core_top | Template Encode Control |
| 93 | core_rx_encode_reg0D | 16 | O | dsp_core_top | Template Encode Control |
| 94 | core_rx_encode_reg0E | 16 | O | dsp_core_top | Template Encode Control |
| 95 | core_rx_encode_reg0F | 16 | O | dsp_core_top | Template Encode Control |
| 96 | core_encode_reg00 | 16 | I | dsp_core_top | Template Encode Result |
| 97 | core_encode_reg01 | 16 | I | dsp_core_top | Template Encode Result |
| 98 | core_encode_reg02 | 16 | I | dsp_core_top | Template Encode Result |
| 99 | core_encode_reg03 | 16 | I | dsp_core_top | Template Encode Result |
| 100 | core_encode_reg04 | 16 | I | dsp_core_top | Template Encode Result |
| 101 | core_encode_reg05 | 16 | I | dsp_core_top | Template Encode Result |
| 102 | core_encode_reg06 | 16 | I | dsp_core_top | Template Encode Result |
| 103 | core_encode_reg07 | 16 | I | dsp_core_top | Template Encode Result |
| 104 | core_encode_reg08 | 16 | I | dsp_core_top | Template Encode Result |
| 105 | core_encode_reg09 | 16 | I | dsp_core_top | Template Encode Result |
| 106 | core_encode_reg0A | 16 | I | dsp_core_top | Template Encode Result |
| 107 | core_encode_reg0B | 16 | I | dsp_core_top | Template Encode Result |
| 108 | core_encode_reg0C | 16 | I | dsp_core_top | Template Encode Result |
| 109 | core_encode_reg0D | 16 | I | dsp_core_top | Template Encode Result |
| 110 | core_encode_reg0E | 16 | I | dsp_core_top | Template Encode Result |
| 111 | core_encode_reg0F | 16 | I | dsp_core_top | Template Encode Result |
| 112 | core_match_start | 1 | O | dsp_core_top | Template Match Start |
| 113 | core_match_tpb_id | 16 | O | dsp_core_top | Template Match Template Index |
| 114 | core_match_finish | 1 | I | dsp_core_top | Template Match Finish |
| 115 | core_match_accept | 1 | I | dsp_core_top | Template Match Accept |
| 116 | core_rx_match_reg00 | 16 | O | dsp_core_top | Template Match Control |
| 117 | core_rx_match_reg01 | 16 | O | dsp_core_top | Template Match Control |
| 118 | core_match_accept_score | 16 | I | dsp_core_top | Template Match Accept Score |
| 119 | core_match_accept_tpb_id | 16 | I | dsp_core_top | Template Match Accept Template Index |
| 120 | core_match_tpb_empty | 1 | I | dsp_core_top | Template Match Template B Empty |
| 121 | core_image_dow_sample | 1 | I | dsp_core_top | Image Read Sample Down Enable 1’b1: Enable 1’b0: Disable |
| 122 | core_image_original_x | 10 | I | dsp_core_top | Image Read Original Point |
| 123 | core_image_original_y | 10 | I | dsp_core_top | Image Read Original Point |
| 124 | core_image_bram_en | 1 | I | dsp_core_top | Image BRAM Read Enable |
| 125 | core_image_bram_addr | 16 | I | dsp_core_top | Image BRAM Read Address(258 * 258) |
| 126 | core_image_bram_rd_data | 8 | O | dsp_core_top | Image BRAM Read Data. Delay 7 Cycle |
| 127 | core_tmpl_bram_en | 1 | O | dsp_core_top | Template BRAM Read Enable |
| 128 | core_tmpl_bram_addr | 9 | O | dsp_core_top | Template BRAM Read Address |
| 129 | core_tmpl_bram_rd_data | 128 | O | dsp_core_top | Template BRAM Read Data |
| 130 | core_match_tpa_we | 1 | O | dsp_core_top | Match.Template A Write Enable |
| 131 | core_match_tpa_addr | 9 | O | dsp_core_top | Match.Template A Write Address |
| 132 | core_match_tpa_wr_data | 128 | O | dsp_core_top | Match.Template A Write Data |
| 133 | core_match_tpb_we | 1 | O | dsp_core_top | Match.Template B Write Enable |
| 134 | core_match_tpb_addr | 9 | O | dsp_core_top | Match.Template B Write Address |
| 135 | core_match_tpb_wr_data | 128 | O | dsp_core_top | Match.Template B Write Data |

  - dsp_ctrl_top模块内部BRAM

    - Image BRAM

Image BRAM用于存储:

- 初始化过程中，由Flash读取的User Code，用于启动CPU；

- 配置过程中，由CPU写入的New User Code；

- 编码过程中，由ISP输入的图像数据，该图像数据由dsp_core_top模块读取，以完成图像编码操作；

Figure Image_BRAM

Image BRAM的大小为147456 * 32 Bit(576K Byte)，由36个16K Byte的SRAM组成。Image BRAM的读数据延时为3 Cycle。其中，读数据通道(dsp_core_top)的读数据延时为7 Cycle，且该通道读取数据大小为256 * 256 * 8 Bit。

AHB Bus只有在DSP处于Idle Status(Register[0x9304]==32‘h81)时，可以对Image BRAM进行读、写操作。

对于Image BRAM，AHB Bus的读、写地址范围为: 32’h3000_0000 ~ 32’h3008_FFFF

    - Template BRAM

Template BRAM用于存储编码得到的模板数据，Template BRAM A/B组成Ping-Pang结构，以加快DSP进行模板识别的速度。

      - Template BRAM A

Template BRAM A用于存储:

- 图像编码后得到的模板数据

- AHB Bus下发的模板数据

- 模板识别操作时，与Template BRAM B组成Ping-Pang结构，存储从Flash中读取的模板数据

Figure Template BRAM A

Template BRAM的大小为2048 * 32 Bit(8K Byte)，读数据延时为3 Cycle。

AHB Bus只有在DSP处于Idle Status(Register[0x9304]==32‘h81)时，可以对Template BRAM A进行读、写操作.

对于Template BRAM A，AHB Bus的读、写地址范围为: 32’h3009_0000 ~ 32’h3009_1FFF

      - Template BRAM B

Figure Template BRAM B

AHB Bus只有在DSP处于Idle Status(Register[0x9304]==32‘h81)时，可以对Template BRAM B进行读、写操作.

对于Template BRAM B，AHB Bus的读、写地址范围为: 32’h3009_2000 ~ 32’h3009_3FFF

      - Match Result BRAM

Match Result BRAM用于存储：

- 在Download Mode下，DSP进行模板识别操作时，得到的全部比对结果

Figure Match Result BRAM

每个模板的比对结果包含Template ID和比对分数，共32 Bit，其中Bit[31:16]为Template ID，Bit[15:0]为比对分数

AHB Bus只有在DSP处于Idle Status(Register[0x9304]==32‘h81)时，可以对Match Result BRAM进行读、写操作.

对于Match Result BRAM，AHB Bus的读、写地址范围为: 32’h3009_4000 ~ 32’h3009_4040

      - DF

  - dsp_apb_ctrl模块

(待续)

  - dsp_ahb_ctrl模块

(待续)

  - dsp_main_ctrl模块

    - dsp_main_ctrl输入输出信号

    - dsp_main_ctrl模块状态图

Figure 12 dsp_main_ctrl模块状态图

    - df

  - dsp_config_ctrl模块

dsp_config_ctrl模块用于完成DSP初始化和配置操作。用户可以根据实际需求，选择对Device Info和Device User Code分别进行配置。

在操作过程中，仅通过Function Bus启动device_info_bus, device_code_bus模块。

    - dsp_config_ctrl模块输入输出信号

Table 3 dsp_config_ctrl模块输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | clk | 1 | I | CRM | dsp working clock |
| 2 | rstn | 1 | I | CRM | dsp working reset |
| 3 | fbus_start_addr_i | 8 | I |  | Function Start Bus Address |
| 4 | fbus_start_data_i | 4 | I |  | Function Start Bus Data |
| 5 | fbus_start_addr_o | 8 | O |  | Function Start Bus Address |
| 6 | fbus_start_data_o | 4 | O |  | Function Start Bus Data |
| 7 | fbus_finish_addr_i | 8 | I |  | Function Finish Bus Address |
| 8 | fbus_finish_data_i | 4 | I |  | Function Finish Bus Data |
| 9 | fbus_finish_addr_o | 8 | O |  | Function Finish Bus Address |
| 10 | fbus_finish_data_o | 4 | O |  | Function Finish Bus Data |

    - dsp_config_ctrl模块状态图

Figure 13 dsp_config_ctrl模块状态图

  - dsp_encode_ctrl模块

dsp_encode_ctrl模块用于采集ISP输入的图像数据并进行编码，在编码成功后，将模板数据存入Template BRAM A中。

在操作过程中，dsp_encode_ctrl模块仅通过Function Bus启动load_image, template_xfer模块。

    - dsp_encode_ctrl模块输入输出信号

Table 3 dsp_encode_ctrl模块输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | clk | 1 | I | CRM | dsp working clock |
| 2 | rstn | 1 | I | CRM | dsp working reset |
| 3 | encrypt_init_start | 1 | O | dsp_core_top | Template Encrypt Initial Start |
| 4 | encrypt_init_finish | 1 | I | dsp_core_top | Template Encrypt Initial Finish |
| 5 | core_encode_rstn | 1 | O | dsp_core_top | Template Encode Software Reset High Active. Hold 32 Cycle |
| 6 | core_encode_start | 1 | O | dsp_core_top | Template Encode Start |
| 7 | core_encode_finish | 1 | I | dsp_core_top | Template Encode Finish |
| 8 | core_encode_valid | 1 | I | dsp_core_top | Template Encode Valid |
| 9 | fbus_start_addr_i | 8 | I |  | Function Start Bus Address |
| 10 | fbus_start_data_i | 4 | I |  | Function Start Bus Data |
| 11 | fbus_start_addr_o | 8 | O |  | Function Start Bus Address |
| 12 | fbus_start_data_o | 4 | O |  | Function Start Bus Data |
| 13 | fbus_finish_addr_i | 8 | I |  | Function Finish Bus Address |
| 14 | fbus_finish_data_i | 4 | I |  | Function Finish Bus Data |
| 15 | fbus_finish_addr_o | 8 | O |  | Function Finish Bus Address |
| 16 | fbus_finish_data_o | 4 | O |  | Function Finish Bus Data |

    - dsp_encode_ctrl模块状态图

Figure 14 dsp_encode_ctrl模块状态图

    - df

  - dsp_accept_ctrl模块

dsp_accept_ctrl模块用于实现模板识别操作。

在操作过程中，仅通过Function Bus启动rx_template_tpa, rx_template_tpb, template_bus, template_management模块。

    - dsp_accept_ctrl模块输入输出信号

Table dsp_accept_ctrl模块输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | clk | 1 | I | CRM | dsp working clock |
| 2 | rstn | 1 | I | CRM | dsp working reset |
| 3 | flash_rstn | 1 | O | ext_flash_top | Flash Software Reset. Low Active |
| 4 | core_match_tpb_empty | 1 | I | dsp_core_top | Match.Template B Empty |
| 5 | core_match_start | 1 | O | dsp_core_top | Template Match Start |
| 6 | core_match_tpb_id_o | 16 | O | dsp_core_top | Template Match Index |
| 7 | core_match_finish | 1 | I | dsp_core_top | Template Match Finish |
| 8 | core_match_accept | 1 | I | dsp_core_top | Template Match Accept |
| 9 | core_match_tpb_id_i | 16 | I | dsp_core_top | Template Matched Index |
| 10 | fbus_start_addr_i | 8 | I |  | Function Start Bus Address |
| 11 | fbus_start_data_i | 4 | I |  | Function Start Bus Data |
| 12 | fbus_start_addr_o | 8 | O |  | Function Start Bus Address |
| 13 | fbus_start_data_o | 4 | O |  | Function Start Bus Data |
| 14 | fbus_finish_addr_i | 8 | I |  | Function Finish Bus Address |
| 15 | fbus_finish_data_i | 4 | I |  | Function Finish Bus Data |
| 16 | fbus_finish_addr_o | 8 | O |  | Function Finish Bus Address |
| 17 | fbus_finish_data_o | 4 | O |  | Function Finish Bus Data |

    - dsp_accept_ctrl模块状态图

Figure dsp_accept_ctrl模块状态图

    - df

  - dsp_remove_ctrl模块

dsp_remove_ctrl模块用于单个或全部模板删除。

在操作过程中，仅通过Function Bus启动template_management，device_info_bus, template_bus模块。

    - dsp_remove_ctrl模块输入输出信号

Table dsp_remove_ctrl模块输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | clk | 1 | I | CRM | dsp working clock |
| 2 | rstn | 1 | I | CRM | dsp working reset |
| 3 | tmpl_clear_en | 1 | I | dsp_apb_ctrl | Template Clear Enable. High Active |
| 4 | tmpl_index_i | 16 | I | dsp_apb_ctrl | Template Index Input |
| 5 | tmpl_index_o | 16 | O | template_bus | Template Index Output |
| 6 | fbus_start_addr_i | 8 | I |  | Function Start Bus Address |
| 7 | fbus_start_data_i | 4 | I |  | Function Start Bus Data |
| 8 | fbus_start_addr_o | 8 | O |  | Function Start Bus Address |
| 9 | fbus_start_data_o | 4 | O |  | Function Start Bus Data |
| 10 | fbus_finish_addr_i | 8 | I |  | Function Finish Bus Address |
| 11 | fbus_finish_data_i | 4 | I |  | Function Finish Bus Data |
| 12 | fbus_finish_addr_o | 8 | O |  | Function Finish Bus Address |
| 13 | fbus_finish_data_o | 4 | O |  | Function Finish Bus Data |

    - dsp_remove_ctrl模块状态图

Figure dsp_remove_ctrl模块状态图

    - df

  - dsp_xfer_ctrl模块

dsp_xfer_ctrl模块用于实现模板输出的读、写操作.

在操作过程中，仅通过Function Bus启动template_bus，template_management，device_info_bus模块

    - dsp_xfer_ctrl模块输入输出信号

Table dsp_xfer_ctrl模块输入输出信号

| No | Name | Width | I/O | Source/ Destination | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | clk | 1 | I | CRM | dsp working clock |
| 2 | rstn | 1 | I | CRM | dsp working reset |
| 3 | tmpl_index_i | 16 | I | dsp_apb_ctrl | Template Index Input |
| 4 | tmpl_index_o | 16 | O | template_bus | Template Index Output |
| 5 | fbus_start_addr_i | 8 | I |  | Function Start Bus Address |
| 6 | fbus_start_data_i | 4 | I |  | Function Start Bus Data |
| 7 | fbus_start_addr_o | 8 | O |  | Function Start Bus Address |
| 8 | fbus_start_data_o | 4 | O |  | Function Start Bus Data |
| 9 | fbus_finish_addr_i | 8 | I |  | Function Finish Bus Address |
| 10 | fbus_finish_data_i | 4 | I |  | Function Finish Bus Data |
| 11 | fbus_finish_addr_o | 8 | O |  | Function Finish Bus Address |
| 12 | fbus_finish_data_o | 4 | O |  | Function Finish Bus Data |

    - dsp_xfer_ctrl模块状态图

Figure dsp_xfer_ctrl模块状态图

    - df

  - fd

- DF